- List the IP address classes (A, B, C, D, E) and give one real-world example of a device or service that might use each class. Write your answers in a table format.

| IP Address Class | Address Range (First Octet) | Default Subnet Mask | Typical Use | Real-World Example |
| --- | --- | --- | --- | --- |
| Class A | 1–126 | 255.0.0.0 (/8) | Very large networks | A large enterprise network (e.g., a multinational company's internal network using 10.0.0.0/8, a private Class A range) |
| Class B | 128–191 | 255.255.0.0 (/16) | Medium-sized organizations | A university or large hospital network using 172.16.0.0/16 (private Class B range) |
| Class C | 192–223 | 255.255.255.0 (/24) | Small business and home networks | A home Wi-Fi router assigning addresses like 192.168.1.x to laptops, phones, and smart TVs |
| Class D | 224–239 | N/A | Multicast communication | IPTV or live video streaming services sending one stream to multiple receivers using multicast |
| Class E | 240–255 | N/A | Experimental and research purposes | Used in networking research and protocol testing; not assigned to normal devices on the public Internet |

- Given the IP address 192.168.10.45 with subnet mask 255.255.255.0, calculate the network address, broadcast address, and the range of valid host addresses.<br><br><em><strong>Hint:</strong> Show your working steps for each calculation.</em>

- IP Address: 192.168.10.45

- Subnet Mask: 255.255.255.0

- CIDR Notation: /24

### Step 1: Determine the Network Portion

Subnet mask 255.255.255.0 means:

- First 24 bits = Network portion

- Last 8 bits = Host portion

IP Address      : 192.168.10.45
Subnet Mask     : 255.255.255.0
Network Address : 192.168.10.0

Calculation (AND operation):

| IP Address | 192 | 168 | 10 | 45 |
| --- | --- | --- | --- | --- |
| Subnet Mask | 255 | 255 | 255 | 0 |
| Result (Network Address) | 192 | 168 | 10 | 0 |

Network Address = 192.168.10.0

### Step 2: Calculate the Broadcast Address

For a /24 network, set all host bits to 1.

Network Address : 192.168.10.0
Host bits       : 11111111 (255)

Broadcast Address = 192.168.10.255

### Step 3: Calculate the Valid Host Range

- First Host = Network Address + 1

  - 192.168.10.1

- Last Host = Broadcast Address − 1

  - 192.168.10.254

### Final Answer

| Item | Value |
| --- | --- |
| IP Address | 192.168.10.45 |
| Subnet Mask | 255.255.255.0 (/24) |
| Network Address | 192.168.10.0 |
| Broadcast Address | 192.168.10.255 |
| First Valid Host | 192.168.10.1 |
| Last Valid Host | 192.168.10.254 |
| Total Addresses | 256 |
| Usable Host Addresses | 254 |

- Create a subnetting plan for a small food delivery startup like Zomato that needs 4 separate subnets: one for Delivery Partners, one for Restaurants, one for Customers, and one for Admins. Assign a unique subnet (with network address and subnet mask) for each group using the 10.0.0.0/24 network.

| Group | Network Address | Subnet Mask | CIDR | Usable Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- | --- | --- |
| Delivery Partners | 10.0.0.0 | 255.255.255.192 | /26 | 10.0.0.1 – 10.0.0.62 | 10.0.0.63 | 62 |
| Restaurants | 10.0.0.64 | 255.255.255.192 | /26 | 10.0.0.65 – 10.0.0.126 | 10.0.0.127 | 62 |
| Customers | 10.0.0.128 | 255.255.255.192 | /26 | 10.0.0.129 – 10.0.0.190 | 10.0.0.191 | 62 |
| Admins | 10.0.0.192 | 255.255.255.192 | /26 | 10.0.0.193 – 10.0.0.254 | 10.0.0.255 | 62 |

- You are given the following scenario: A college WiFi network uses the IP range 172.16.0.0/16. The admin wants to create at least 20 subnets for different buildings. Calculate the new subnet mask and the number of hosts available per subnet.<br><br><em><strong>Constraint:</strong> Show your calculation steps and final answers.</em>

### Step 1: Calculate the Number of Borrowed Bits

Use the formula:

2n≥Required Subnets2^n \geq \text{Required Subnets}2n≥Required Subnets

Where n = number of bits borrowed.

- 24=162^4 = 1624=16 ❌ (Not enough)

- 25=322^5 = 3225=32 ✅ (Enough)

So, borrow 5 bits.

### Step 2: Calculate the New Prefix Length

Original prefix = /16

Borrowed bits = 5

New prefix:

16+5=2116 + 5 = 2116+5=21

New Prefix = /21

### Step 3: Determine the New Subnet Mask

A /21 subnet mask is:

- Binary:

11111111.11111111.11111000.00000000

- Decimal:

255.255.248.0

New Subnet Mask = 255.255.248.0 (/21)

### Step 4: Calculate Hosts per Subnet

Total host bits:

32−21=1132 - 21 = 1132−21=11

Usable hosts:

211−2=2048−2=20462^{11} - 2 = 2048 - 2 = 2046211−2=2048−2=2046

Hosts available per subnet = 2046

### Step 5: Verify Number of Subnets

Borrowed bits = 5

25=32 subnets2^5 = 32 \text{ subnets}25=32 subnets

This satisfies the requirement of at least 20 subnets.

### Final Answers

| Item | Value |
| --- | --- |
| Original Network | 172.16.0.0/16 |
| Required Subnets | At least 20 |
| Borrowed Bits | 5 |
| New Prefix | /21 |
| New Subnet Mask | 255.255.248.0 |
| Total Subnets Created | 32 |
| Host Bits | 11 |
| Usable Hosts per Subnet | 2046 |

- A friend sends you a message asking why their device with IP 192.168.1.300 is not connecting to the network. Write a short explanation identifying the error and what a valid IP address in that subnet should look like.

The IP address 192.168.1.300 is invalid because each octet in an IPv4 address must be a number between 0 and 255. Since 300 is greater than 255, the device cannot use this address and will not connect to the network.

For the 192.168.1.0/24 subnet, a valid host IP address should be in the range 192.168.1.1 to 192.168.1.254. The address 192.168.1.0 is the network address, and 192.168.1.255 is the broadcast address, so neither can be assigned to a device.

Examples of valid IP addresses in this subnet include 192.168.1.10, 192.168.1.50, and 192.168.1.200.